YOCTO BUILD GUIDE

RK3288 EVB — Scarthgap (Poky 5.0)

Build 1: core-image-minimal

Build 2: rk3288-terminal-image (HDMI Display)

March 2026

Kernel: Linux 6.1  |  U-Boot: 2017.09  |  Distro: Poky 5.0.15

# 1. Project Overview

This document covers the complete process of building two Yocto images for the Rockchip RK3288 EVB board using the Poky reference distribution on the Scarthgap (5.0) release. It documents every step taken, the reason behind each decision, all bugs encountered, and exactly how they were resolved.

| Item | Value |
| --- | --- |
| Host OS | Ubuntu (x86_64) |
| Yocto Release | Scarthgap (Poky 5.0.15) |
| Target Board | Rockchip RK3288 EVB |
| Machine Config | rockchip-rk3288-evb |
| Kernel Version | Linux 6.1 (linux-rockchip_6.1.bb) |
| U-Boot Version | 2017.09 |
| Init System | systemd |
| Build 1 Image | core-image-minimal |
| Build 2 Image | rk3288-terminal-image (HDMI terminal) |
| Build Directory | /nvme/yocto/poky/build |

# 2. Layer Setup & Configuration

## 2.1 Layers Used

The following BSP and middleware layers were added to the build. Each serves a specific purpose:

| Layer | Purpose |
| --- | --- |
| meta | Poky core — base recipes, classes, toolchain |
| meta-poky | Poky distro configuration |
| meta-yocto-bsp | Reference BSP machines |
| meta-arm / meta-arm-toolchain | ARM architecture support and GCC toolchain |
| meta-openembedded/meta-oe | Extended package recipes (htop, i2c-tools, etc.) |
| meta-rockchip | Rockchip SoC BSP — kernel, U-Boot, Mali GPU, ISP drivers |

## 2.2 Why These Layers Are Needed

meta-rockchip is the critical layer. Without it, Yocto has no knowledge of the RK3288 SoC, its memory map, boot sequence, device tree, or any Rockchip-specific drivers. It provides the machine configuration file (rockchip-rk3288-evb.conf) and the kernel recipe (linux-rockchip_6.1.bb) that describe exactly how to build for this hardware.

meta-openembedded/meta-oe is required because many practical packages (SSH server, htop, i2c-tools) are not in the Poky core. meta-arm provides the ARM Cortex-A17 toolchain tuning used by the RK3288.

## 2.3 Machine Selection

Three RK3288 machine configs exist in meta-rockchip. The correct one was identified by inspecting the meta-rockchip/conf/machine/ directory:

| Machine Config File | Use Case |
| --- | --- |
| rockchip-rk3288-evb.conf | Standard RK3288 EVB — used for this build |
| rockchip-rk3288-evb-act8846.conf | EVB variant with ACT8846 PMIC chip |
| rockchip-rk3288w-evb.conf | RK3288W low-power variant (tablet-focused) |

The standard rockchip-rk3288-evb was selected as it covers the majority of EVB boards. The MACHINE variable in local.conf was set to match exactly: MACHINE = "rockchip-rk3288-evb".

# 3. Build 1 — core-image-minimal

## 3.1 Goal

Produce the smallest possible bootable Linux image to validate the toolchain, BSP, kernel, and U-Boot for the RK3288 EVB. This is standard practice when bringing up a new board — start minimal, confirm it boots, then add features.

## 3.2 local.conf Settings

The following settings were configured in build/conf/local.conf before starting Build 1:

MACHINE = "rockchip-rk3288-evb"

DISTRO = "poky"

# systemd as init manager (requires usrmerge in Scarthgap)

DISTRO_FEATURES:append = " systemd usrmerge"

DISTRO_FEATURES:remove = "sysvinit"

VIRTUAL-RUNTIME_init_manager = "systemd"

VIRTUAL-RUNTIME_initscripts = "systemd-compat-units"

# Parallel build tuning

BB_NUMBER_THREADS = "8"

PARALLEL_MAKE = "-j8"

EXTRA_IMAGE_FEATURES += "debug-tweaks ssh-server-openssh"

## 3.3 Build Command

bitbake core-image-minimal

## 3.4 Bug #1 — systemd Missing usrmerge Feature

ERROR: Nothing RPROVIDES 'systemd' systemd was skipped: missing required distro feature 'usrmerge' (not in DISTRO_FEATURES)

### Root Cause

In Scarthgap (Yocto 5.0), systemd mandates the usrmerge distro feature as a hard dependency. UsrMerge is a Linux standard that merges /bin, /sbin, and /lib into their /usr equivalents (/usr/bin, /usr/sbin, /usr/lib). Older Yocto releases (e.g. Kirkstone) did not enforce this. Scarthgap strictly enforces it.

Because DISTRO_FEATURES only contained 'systemd' but not 'usrmerge', the systemd recipe was marked as skipped, causing a cascade: packagegroup-core-boot requires systemd, and core-image-minimal requires packagegroup-core-boot — making the entire image unbuildable.

### Fix Applied

Added 'usrmerge' to DISTRO_FEATURES alongside 'systemd' in local.conf:

DISTRO_FEATURES:append = " systemd usrmerge"

DISTRO_FEATURES:remove = "sysvinit"

VIRTUAL-RUNTIME_init_manager = "systemd"

VIRTUAL-RUNTIME_initscripts = "systemd-compat-units"

## 3.5 Bug #2 — fiq_debugger Kernel Compile Error

ERROR: fiq_debugger_arm.c:241 — implicit declaration of function 'THREAD_INFO' drivers/soc/rockchip/fiq_debugger/fiq_debugger_arm.c: error: implicit declaration of function 'THREAD_INFO' [-Werror=implicit-function-declaration]

### Root Cause

The Rockchip FIQ Debugger driver (drivers/soc/rockchip/fiq_debugger/) uses the THREAD_INFO() macro to retrieve thread_info from a stack pointer. This macro was removed from the Linux kernel in version 4.9+ when the thread_info struct was moved out of the kernel stack into task_struct. The Rockchip BSP driver code was never updated to reflect this change.

Because the kernel is built with -Werror (warnings treated as errors), this implicit function declaration causes a hard compile failure. The driver fails to build, causing the entire kernel build to abort.

### Fix Applied

Rather than patching the driver source (which would require ongoing maintenance as the kernel evolves), the cleaner approach was to disable the FIQ Debugger via a kernel config fragment. The FIQ Debugger is a low-level UART crash debugger — not needed for normal EVB bring-up.

Two files were created:

File 1 — Kernel config fragment:

Path: meta-rockchip/recipes-kernel/linux/linux-rockchip/disable-fiq-debugger.cfg

CONFIG_FIQ_DEBUGGER=n

CONFIG_RK_CONSOLE_THREAD=n

File 2 — bbappend to inject the fragment:

Path: meta-rockchip/recipes-kernel/linux/linux-rockchip_6.1.bbappend

FILESEXTRAPATHS:prepend := "${THISDIR}/${PN}:"

SRC_URI:append = " file://disable-fiq-debugger.cfg"

After applying the fix, the kernel was cleaned and rebuilt:

bitbake linux-rockchip -c cleansstate

bitbake core-image-minimal

## 3.6 Build 1 Result

Build 1 SUCCESS NOTE: Tasks Summary: Attempted 4660 tasks of which 2968 didn't need to be rerun and all succeeded.

| Output File | Purpose |
| --- | --- |
| boot.img | Kernel + DTB + initramfs combined boot image |
| uboot.img | U-Boot bootloader |
| trust.img | ARM Trusted Firmware (TF-A / OP-TEE) |
| idblock.img | DDR initialization + miniloader (first stage) |
| loader.bin | U-Boot TPL/SPL loader |
| rk3288-evb-rk808-linux.dtb | Device tree blob |
| core-image-minimal-...tar.gz | Root filesystem (17 MB) |
| modules-...tgz | Kernel modules |

# 4. Build 2 — rk3288-terminal-image (HDMI Display)

## 4.1 Goal

Extend Build 1 to produce a more capable image that shows a login terminal directly on the HDMI-connected display. This is useful for headless-less development, demos, and scenarios where serial console access is not convenient. The board's HDMI output is driven by the Synopsys DesignWare HDMI controller integrated into the RK3288 SoC.

## 4.2 Changes Made vs Build 1

### 4.2.1 local.conf additions

The following lines were appended to local.conf to enable X11 display infrastructure and additional packages:

# Display support

DISTRO_FEATURES:append = " x11"

# Extra packages installed into image

IMAGE_INSTALL:append = " kbd util-linux bash openssh openssh-sshd"

# Developer-friendly image features

EXTRA_IMAGE_FEATURES += "debug-tweaks ssh-server-openssh tools-debug"

x11 was added to DISTRO_FEATURES because the DRM/KMS display stack on Rockchip depends on it being present. Without it, certain display-related packages are excluded from the build by conditional DISTRO_FEATURES checks in their recipes.

### 4.2.2 Kernel config fragment — HDMI and framebuffer drivers

The existing disable-fiq-debugger.cfg was updated to also enable the kernel drivers required for HDMI output and framebuffer console:

# Disable broken driver (from Build 1)

CONFIG_FIQ_DEBUGGER=n

CONFIG_RK_CONSOLE_THREAD=n

# DRM / Rockchip display engine

CONFIG_DRM=y

CONFIG_DRM_ROCKCHIP=y

CONFIG_FB=y

CONFIG_FRAMEBUFFER_CONSOLE=y

CONFIG_VT=y

CONFIG_VT_CONSOLE=y

CONFIG_DUMMY_CONSOLE=y

# Synopsys DesignWare HDMI controller (integrated in RK3288)

CONFIG_DRM_DW_HDMI=y

CONFIG_DRM_DW_HDMI_AHB_AUDIO=y

CONFIG_DRM_DW_HDMI_I2S_AUDIO=y

CONFIG_ROCKCHIP_DW_HDMI=y

| Config Option | Why It Is Needed |
| --- | --- |
| CONFIG_DRM_ROCKCHIP | Main Rockchip DRM/KMS display driver — controls the VOP (Video Output Processor) |
| CONFIG_DRM_DW_HDMI | Synopsys DesignWare HDMI IP driver — the HDMI encoder block inside the RK3288 |
| CONFIG_ROCKCHIP_DW_HDMI | Rockchip-specific glue between DRM_ROCKCHIP and DRM_DW_HDMI |
| CONFIG_FRAMEBUFFER_CONSOLE | Renders the Linux text console onto the framebuffer (what you see on screen) |
| CONFIG_VT / CONFIG_VT_CONSOLE | Virtual Terminal subsystem — required for getty and login prompt on tty1 |

### 4.2.3 Custom image recipe

A new BitBake image recipe was created to define the exact package set for the terminal image:

Path: meta-rockchip/recipes-core/images/rk3288-terminal-image.bb

SUMMARY = "RK3288 image with HDMI terminal display"

require recipes-core/images/core-image-base.bb

IMAGE_INSTALL:append = " \

kbd \

util-linux \

bash \

openssh \

openssh-sshd \

i2c-tools \

htop \

e2fsprogs \

"

IMAGE_FEATURES += " \

debug-tweaks \

ssh-server-openssh \

tools-debug \

"

core-image-base was used as the base (rather than core-image-minimal) because it includes a broader set of filesystem utilities and init scripts that are needed when a display and input subsystem is present.

### 4.2.4 systemd getty on tty1 — login prompt on HDMI

By default, systemd only starts getty on ttyFIQ0 (the Rockchip serial console). To show a login prompt on the HDMI display, getty needs to be started on tty1 (the framebuffer virtual terminal). This was done via a systemd service override:

File 1 — getty override config:

Path: meta-rockchip/recipes-core/systemd/systemd/tty1-getty.conf

[Service]

ExecStart=

ExecStart=-/sbin/agetty --autologin root --noclear tty1 linux

File 2 — systemd bbappend to install the override:

Path: meta-rockchip/recipes-core/systemd/systemd_%.bbappend

FILESEXTRAPATHS:prepend := "${THISDIR}/${PN}:"

SRC_URI:append = " file://tty1-getty.conf"

do_install:append() {

install -d ${D}${systemd_system_unitdir}/getty@tty1.service.d/

install -m 0644 ${WORKDIR}/tty1-getty.conf \

${D}${systemd_system_unitdir}/getty@tty1.service.d/override.conf

install -d ${D}${systemd_system_unitdir}/getty.target.wants/

ln -sf ../getty@.service \

${D}${systemd_system_unitdir}/getty.target.wants/getty@tty1.service

}

The --autologin root flag means the board will automatically log in as root on boot and show a shell prompt on the HDMI display without requiring a password — appropriate for a development EVB.

## 4.3 Build Command

bitbake linux-rockchip -c cleansstate

bitbake rk3288-terminal-image

# 5. Flashing the Board

## 5.1 Output Files

Both builds produce the same set of boot images (bootloader, kernel, DTB). The rootfs tarball differs between builds. The deploy directory is:

/nvme/yocto/poky/build/tmp/deploy/images/rockchip-rk3288-evb/

## 5.2 Why No .wic File?

Unlike PC-style boards, Rockchip platforms use a proprietary partition layout and boot ROM protocol. The meta-rockchip BSP produces individual partition images (idblock, uboot, trust, boot, rootfs) rather than a single .wic SD card image. Each partition must be written to a specific LBA offset.

## 5.3 Flashing via rkdeveloptool (Maskrom Mode)

Put the board into Maskrom mode by holding the RECOVERY button while powering on, then connect the USB OTG port to the host PC.

cd /nvme/yocto/poky/build/tmp/deploy/images/rockchip-rk3288-evb/

# Step 1: Initialize DDR via loader

sudo rkdeveloptool db loader.bin

# Step 2: Flash partitions in order

sudo rkdeveloptool wl 0x0 idblock.img # DDR init + miniloader

sudo rkdeveloptool wl 0x40 uboot.img # U-Boot

sudo rkdeveloptool wl 0x1000 trust.img # ARM Trusted Firmware

sudo rkdeveloptool wl 0x2000 boot.img # Kernel + DTB

# Step 3: Reboot

sudo rkdeveloptool rd

## 5.4 Monitoring Boot via Serial Console

# 115200 baud on ttyFIQ0 (as defined in rk3288.inc)

sudo picocom /dev/ttyUSB0 -b 115200

# 6. Bug Summary

| Bug | Error | Root Cause | Fix |
| --- | --- | --- | --- |
| Bug #1 Build 1 | Nothing RPROVIDES 'systemd' | Scarthgap enforces 'usrmerge' as a hard dependency for systemd. Was not required in older releases like Kirkstone. | Added 'usrmerge' to DISTRO_FEATURES alongside 'systemd' in local.conf |
| Bug #2 Build 1 | fiq_debugger_arm.c: implicit declaration of THREAD_INFO() | THREAD_INFO() macro removed from Linux kernel 4.9+. Rockchip BSP driver was never updated. -Werror makes it a hard failure. | Created kernel .cfg fragment to set CONFIG_FIQ_DEBUGGER=n and injected via bbappend |

# 7. Files Created / Modified

| File Path | Purpose |
| --- | --- |
| build/conf/local.conf | Machine, distro, systemd, display, and package settings |
| meta-rockchip/recipes-kernel/linux/linux-rockchip_6.1.bbappend | Injects kernel config fragment into linux-rockchip 6.1 build |
| meta-rockchip/recipes-kernel/linux/linux-rockchip/disable-fiq-debugger.cfg | Disables FIQ debugger; enables DRM, framebuffer, HDMI kernel options |
| meta-rockchip/recipes-core/images/rk3288-terminal-image.bb | Custom image recipe with SSH, kbd, htop, bash, and display packages |
| meta-rockchip/recipes-core/systemd/systemd_%.bbappend | Installs tty1 getty override so login prompt appears on HDMI display |
| meta-rockchip/recipes-core/systemd/systemd/tty1-getty.conf | systemd drop-in: starts agetty with autologin on tty1 (framebuffer) |

# 8. Key Design Decisions

| Decision | Reason |
| --- | --- |
| Used kernel 6.1 (not 4.4 or 5.10) | rockchip-common.inc sets PREFERRED_VERSION_linux-rockchip = "6.1%" for all Rockchip machines on Scarthgap. It is the maintained LTS kernel for this BSP. |
| Disabled FIQ Debugger (not patched) | Disabling is safer and lower maintenance than patching a driver that touches kernel-internal structures that change frequently across versions. |
| Used core-image-base for Build 2 | core-image-minimal lacks some filesystem and init infrastructure needed when a display subsystem is active. core-image-base is a safer foundation. |
| Autologin on tty1 | Development EVB does not need login security. --autologin root gives immediate shell access on screen after boot, speeding up development iteration. |
| Individual partition flashing (not .wic) | meta-rockchip BSP does not generate .wic images. Rockchip boot ROM requires specific LBA offsets for each partition, which rkdeveloptool handles directly. |

End of Document — RK3288 EVB Yocto Build Guide